Team 3 – Executive Summary

Healthy Body Vitamins (HBV) manufactures and distributes over-the-counter vitamin supplements. Their specialty is sports nutrition and weight management products. They are also a manufacturer of generic versions of a few of their best sellers and these products are sold as Vital Brand TM. Customers purchase their products through company retail stores, over the phone, or online.

The goal of this project is to design a new facility that effectively and efficiently integrates material handling, storage, and product flow to meet the future needs of HBV, and that also includes manufacturing space for Vital Brand. There are many problems that the team as a whole needed to solve. This involved deciding between 173,000 vs 215,000 square ft facility size, 12 vs 16 dock doors, how to include the parking and corporate offices, and how to make sure the facility was properly designed for the projected 5 years of growth.

There is a substantial uptick in sales predicted for HBV in between year 0 when the factory is built to year 5, the final design year. In order to properly design a facility that will be able to meet these requirements, multiple factors are to be considered. The facility needs to be able to take in and store the necessary raw materials, accommodate enough space to manufacture the desired goods, have proper warehouse storage, and include the necessary accommodation for employees. Using a SolidWorks drawing and the predicted values for year 5, the team was able to conclude that a 173,000 square ft facility would be sufficient to meet the needs of HBV. This is an important conclusion to come to as the 215,000 square ft facility costs 2.1 million more dollars to build.

There are many things that need to be considered when designing a facility. For this particular scenario, the facility also needs to house an outlet store and corporate offices. The actual interior of these two places in the facility are to be designed by a different contractor, however we do know that the combined area of these would only take up 28,000 square ft, so there would be plenty of room for the manufacturing and warehouse storage. The warehouse storage section was designed with the needs of facility employees in mind. When deciding where the storage racks will be, the team considered the required lane size for forklift logistics to be 13 feet wide. The racks in the warehouse are going to be 3.5 ft deep.

Price ticketing is recommended to occur after the product has finished the picking process. Currently, HBV places the price tickets on products inside the company stores, however, it is determined that this costs $1.25 million a year in employee labor costs. Doing the price ticketing after picking in the facility will make this process more streamlined and reduce the overall cost to the company.

The conclusion was reached that 12 dock doors would be sufficient to meet the shipping needs of HBV. This was a major deciding variable as any more than 12 dock doors would increase the cost of the facility and if the team wanted 16 dock doors, it would be difficult to acquire the necessary construction permits from the city for such an addition. Close to the dock doors is the shipping and receiving section which leads straight to raw material storage, forward area storage, and warehouse storage.

In order to properly decide how many units, the facility will have to produce for HBV, the demand for the 5 years needs to be calculated. With that information, the team is able to tackle the manufacturing requirements of the facility. This demand was calculated using the projected Vital Brand sales in combination with the percentage of total sales that vital brand products make up.

HBV uses filling lines in the manufacturing process in order to fill empty bottles with specific vitamins and supplements. More lines can be added, or current lines can be upgraded to meet production needs. A new high-speed line can be added in for the cost of $1.65 million, or an existing line can be upgraded to a high-speed line for the cost of $940,000. The team determined that an optimal decision for HBV would be to have just the 2 existing lines for year 0, then line 1 will be upgraded to a high-speed line for year 1, and that will maintain significant production until year 3 where an additional line will be upgraded to a high-speed line. With these upgrades, HBVs supply will be able to keep up with the demand while spending the least amount of capital. This was calculated using the predicted future demand and the number of units produced per hour for each type of line.

The team decided to use Arena simulation software to simulate the inner workings of the facility and make sure that the design was optimal for the 5-year projected growth of HBV. This model allows for a basic attempt at replicating a warehouse’s operation function. It utilized department flow timings and distributions to examine the number of employees needed for operation. The model indicates that 4 employees are necessary to functionally run the warehouse so long as receiving and shipping are timed properly. The model also allows for a look at the production of Vital Brand products. It showed that so long as the lines are not swapped more than twice a day the demand can be supplied.

Our recommendation for the Healthy Body Vitamins corporations is to use the 173,000 square ft facility with 12 dock doors. After year 0, line 1 should be upgraded in the facility. After year 3, a brand-new high-speed line should be added into the facility. Consequently, they can exceed demand at the end of year 5, however it is getting concerningly close to the total supply. If they wish for a bit safer of a range the final line can be upgraded.

Recommendations for future study involve a more in-depth investigation into the specific workings of the factory. An example of this is employees in the facility. Is it beneficial to have more employees in the facility for certain days? Do they need to work overtime? Are they going to receive a pay increase for each year? As this is a substantial cost for the company, taking a further look into these issues is certain to be beneficial. Another future study recommendation would be to look more in depth into warehouse storage. The majority of the facility is taken up by storage space, so there is a lot that goes into organization, forklift travel, and SKU picking. Also, only rough estimates based on similar products were used to get pricing information to give us a general idea of the capital input needed for the facility equipment. A more precise study of the facilities needs would provide a more accurate price point for the sponsor.